SIMATS ENGINEERING

Department of Computer Science and Engineering

ASSESSMENT TOOL 1 – ANALYTICAL PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 – Precision (BL3) | Assessment: | Analytical Problem Solving |
| Weightage: | 45% | Total Marks: | 100 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. |  |  |
| Student Name: | G LOKESH YADAV | Reg. No.: | 192411061 |
| Section / Batch: | CSE-2024 | Date: | 03-08-2024 |

Instructions to Students • Identify the appropriate arithmetic algorithm or number representation required for the given computational problem. • Analyse the step-by-step execution of integer and floating-point arithmetic operations using the selected algorithm. • Apply Booth's multiplication, restoring/non-restoring division, and IEEE 754 standards to obtain accurate results. • Compute binary multiplication, division, normalization, and floating-point arithmetic using appropriate algorithms. • Interpret the computed results by evaluating their accuracy, precision, and suitability for the given application.

QUESTIONS

1. A processor performs arithmetic operations on signed 8-bit integers using 2’s complement representation.  Consider two numbers: +45 and −23. Analyse in detail how the system performs both addition and  subtraction, including binary conversion, alignment of sign bits, and intermediate steps. Determine whether  overflow occurs, and explain how the processor detects overflow and interprets the final result.

2. A high-performance computing system replaces a ripple carry adder with a 4-bit carry look-ahead adder  (CLA) to reduce computation delay. Given two binary inputs, analyze how generate and propagate signals  are formed and how carries are computed in advance. Compare the time delay of CLA with ripple carry  addition and justify, with reasoning, why CLA improves speed in modern processors.

3. In a signed multiplication operation, a processor uses Booth’s algorithm to multiply two numbers, −6 and  +5. Analyze the algorithm step-by-step, including the role of the accumulator, multiplier, and Booth bit,  along with arithmetic shifts. Explain how the algorithm efficiently handles signed numbers and reduces the  number of addition/subtraction operations compared to traditional multiplication.

4. A processor is required to perform division of two binary numbers (e.g., 13 ÷ 3) using both restoring and  non-restoring division algorithms. Analyze both methods step-by-step, showing intermediate remainders  and quotient formation. Compare the two approaches in terms of number of steps, complexity, and  efficiency, and explain why non-restoring division is often preferred in modern systems.

5. A scientific application requires precise handling of real numbers using the IEEE 754 standard for floating point representation. Given a decimal number (e.g., −13.25), analyse how it is represented in both single  precision and double precision formats, including sign, exponent, and mantissa fields. Further, explain how  floating-point addition is performed between two such numbers, highlighting issues like normalization,  rounding, and precision loss.

Code of Conduct

I G LOKESH YADAV -192411061 (Name / Reg No) certify that this submission is my original work and  that I have adhered to the guidelines specified for this assessment. I understand that any violation of  academic integrity rules will result in disciplinary action.

Signature of the Student: Lokesh yadav

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 – Exemplary (5 Marks) | Level 3 – Proficient (4 Marks) | Level 2 – Developing (2–3 Marks) | Level 1 – Beginning (0–1 Mark) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |

Marks Evaluation:

| Q. No | Problem Understanding (20) |  | Method Selection (20) | Solution Process (25) | Accuracy of Calculation (20) | Interpretation of Results (15) | Total (out of 100) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 2 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 3 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 4 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 5 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
|  | / 20 |  | / 20 | / 25 | / 20 | / 15 | / 100 |

Course Coordinator Faculty Incharge

Q1. Signed 8-bit Addition and Subtraction

Given A=+45, B=-23

+45 = 00101101
+23 = 00010111
1's complement =11101000
2's complement =11101001 (-23)

Addition:
 00101101
+11101001
----------
100010110
Ignore carry => 00010110 = 22
Overflow: No (operands have different signs)

Subtraction:

45-(-23)=45+23

 00101101
+00010111
----------
 01000100 = 68
Overflow: No.

Explanation:
• Positive numbers are stored directly in binary.
• Negative numbers are represented using 2's complement.
• Overflow occurs only when two numbers of the same sign produce a result of opposite sign. Since the results are within -128 to +127, no overflow occurs.

Q2. Carry Look-Ahead Adder

Carry Look-Ahead Adder
Example: A=1011, B=1101, C0=0

P=A⊕B = 0110
G=A·B =1001

C1=G0+P0C0
C2=G1+P1G0+P1P0C0
C3=G2+P2G1+P2P1G0+P2P1P0C0
C4=G3+P3G2+P3P2G1+P3P2P1G0+P3P2P1P0C0

| Term | Formula | Purpose |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Propagate | P=A⊕B | Propagates carry |  |  |  |
| Generate | G=A·B | Generates carry |  |  |  |
| C1 | G0+P0C0 | Carry 1 |  |  |  |
| C2 | G1+P1G0+P1P0C0 | Carry 2 |  |  |  |

Explanation:
• CLA calculates all carry bits simultaneously.
• Ripple Carry Adder waits for each carry sequentially.
• Hence CLA has much lower propagation delay and is used in high-speed processors.

Q3. Booth Multiplication

Booth's Multiplication
Multiply -6 × +5

M=-6=11111010
Q=00000101
A=00000000 Q-1=0

Apply Booth rules for 8 cycles:
01 -> A=A+M
10 -> A=A-M
00/11 -> No operation
Arithmetic right shift after each cycle.

Final Product = 11100010 = -30

| Cycle | Q0Q-1 | Operation | Shift | Remark |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | A=A-M | ARS | Subtract |  |  |  |  |  |
| 2 | 01 | A=A+M | ARS | Add |  |  |  |  |  |
| 3-8 | 00/11 | No Operation | ARS | Continue |  |  |  |  |  |

Explanation:
• Booth's algorithm efficiently multiplies signed numbers.
• It reduces unnecessary additions for consecutive 1's, improving speed.

Q4. Restoring and Non-Restoring Division

Restoring Division Example: 13/3

Dividend=1101
Divisor=0011

Shift left, subtract divisor.
If result negative, restore remainder and set quotient bit=0.
Else quotient bit=1.

Final Quotient=0100 (4)
Remainder=0001 (1)

Explanation:
• Restoring division restores the previous remainder whenever subtraction becomes negative.
• Non-restoring division skips this restoration during each iteration, making it faster and requiring fewer operations.

Q5. IEEE-754 Floating Point

IEEE-754 Representation of -13.25

13.25 = 1101.01₂
Normalized =1.10101×2^3

Single Precision:
Sign=1
Exponent=3+127=130=10000010
Mantissa=10101000000000000000000

32-bit:
1 10000010 10101000000000000000000

Double Precision:
Exponent=3+1023=1026=10000000010
Mantissa=1010100000000000000000000000000000000000000000000000

| Field | Single Precision | Meaning |  |  |
| --- | --- | --- | --- | --- |
| Sign | 1 | Negative |  |  |
| Exponent | 10000010 | Bias 127 |  |  |
| Mantissa | 10101000000000000000000 | Fraction |  |  |

Explanation:
• IEEE-754 stores a floating-point number using three fields: Sign, Exponent, and Mantissa.
• The exponent is stored using a bias (127 for single precision, 1023 for double precision).
• Normalization improves precision and ensures a unique representation.